| Artigo S/N: Item S/N: | Artigo P/N: Item P/N: | Manual: Manual: | Overhaul Manual GTP5243-3 – 72-60-22 |
| --- | --- | --- | --- |
| Operador: Customer: | TLN: 2118 | Edição: Issue: | E1R24-72-2 |
|  |  | Data: Date: | Aug. 12/2022 |

| Operação Sequence | Referência Reference | Procedimentos / Tolerâncias Procedures / Serviceable Limits | Executado Executed by | Inspecionado Inspected by |
| --- | --- | --- | --- | --- |
| 10 | Pag. 101 2.A | Coloque a gearbox no suporte 6795579, remova parafusos, porcas e arruelas, separe a tampa da carcaça e tampe os tubos de fornecimento de óleo. Place the gearbox on 6795579 stand, remove bults, nuts and washers, separate the housing from cover and plug the oil supply pipes. |  |  |
| 20 | Pag. 101 2.A | Gearbox Cover Disassembly Efetue os passos (1) a (3). Retire o injetor de óleo do rolamento de pinhão da tampa da gearbox. Remova o suporte do eixo do torquímetro, ao mesmo tempo, remova o “helical power takeoff gershaft” e “helical torquemeter gearshaft”. Do the step (1) to (3). take the pinion bearing oil nozzle from the gearbox cover. Remove the torquemeter shaft support from the gearbox cover, simultaneously remove the helical power takeoff gearshaft and the helical torquemeter gearshaft. |  |  |
| 30 |  | Efetue o passo (4). Remova a pista exterior do rolamento e roletes do “helical torquemeter gearshaft”. Do the Step (4). Remove the bearing outer ring and rollers from “helical torquemeter gearshaft. |  |  |
| 40 | Pag. 101 2.A | Executar passos (5) e (6). Remova o rolamento esférico do “helical power takeoff gearshaft” e a pista interna do rolete de rolamento. Remova o anel interno de retenção e o “power train drive helical gear” da tampa da gearbox. Do the Steps (5) and (6). Remove the ball bearing from the helical power takeoff gearshaft and the roller bearing inner race. Remove the internal retaining ring power train drive helical gear from the gearbox cover. |  |  |
| 50 | Pag. 103 2.A | Executar os passos (7). Remova o anel retentor interno e rolete de rolamento (No. 2-1/2) de dentro do “helical drive gear”. Do the step (7). Remove the internal retaining ring and roller bearing (No. 2-1/2) from inside the helical drive gear. |  |  |
| 60 | Pag. 103 2.A | Efetue passo (8). Remova o “fuel control spur gearshaft” da tampa. Remova os dois rolamentos e o rolamento espaçador do eixo. Do the step (8). Remove the fuel control spur gearshaft from the cover. Remove the two bearings and the bearing spacer from the shaft. |  |  |
| 70 | Pag. 103 2.A | Efetue passo (9). Remova o anel externo de retenção e o retentor de rolamento. Remova os dois rolamentos e o rolamento espaçador do eixo. NOTA: Não remova os plugs do “spur gearshaft” a menos que estejam frouxos. Do the step (9). Remove the two bearings and the bearing spacer from the shaft. NOTE: Do not remove the plugs from the spur gearshafts unless they are loose. |  |  |
| 80 | Pag. 104 2.A | Efetue o passo (10). Remova o “accessory” drive spur gearshaft” (reserva) da tampa da gearbox. Remova o anel interno de retenção e o retentor do rolamento. Remova os rolamentos e o espaçador do eixo. Do the step (10). Remove the accessory drive spur gearshaft (spare) from the gearbox cover. Remove the internal retaining ring and the bearing retainer. Remove the bearings and the spacer from the shaft. |  |  |
| 90 | Pag. 104 2.A | Efetue o passo (11). Remova o “starter generator spur gearshaft” da tampa da gearbox. Remova o anel externo de retenção e o retentor do rolamento. Remova os rolamentos e o espaçador do eixo. Do the step (11). Remove the starter generator spur gearshaft from gearbox cover. Remove the external retaining ring and the bearing retainer. Remove the bearings and the spacer from the shaft. |  |  |
| 100 | Pag. 104 2.A | Efetue o passo (12). Remova o “fuel pump drive spur gearshaft” da tampa da gearbox. Remova o anel externo de retenção e o retentor do rolamento. Remova os rolamentos e o espaçador do eixo. Do the step (12). Remove the fuel pump drive spur gearshaft from the gearbox cover. Remove the external retaining ring and the bearing retainer. Remove the bearings and the spacer from the shaft. |  |  |
| 110 | Pag. 105 2.A | Efetue passo (13). Remova os dois “gas producer train idler spur gears” expostos. Remova a engrenagem e o rolamento do “idler gear support shaft”. NOTA: Os rolamentos estão presos por um anel interno de retenção Do the step (13). Remove the two exposed gas producer train idler spur gears. Remove the gear and bearing from the Idler Gear Support Shaft. NOTE: The bearing is retained by an internal ring |  |  |
| 120 | Pag. 105 2.A | Efetue o passo (14). Desbloqueie o “gas producer train idler spur gearshaft” e remova os elementos de retenção. Remova o anel externo de retenção e o retentor do rolamento. Remova os rolamentos Do the step (14). Unlock the key washer of the gas producer train idler spur gear and remove the retaining elements. Remove the external retaining ring and the bearing retainer. Remove the bearing |  |  |
| 130 |  | Efetue o passo (15). Remova a porca e arruela do “gas producer train idler spur gear” abaixo do “gas producer train idler spur gearshaft”. Remova o “Idler Spur Gear”, parafusos e arruelas do seu eixo de suporte enquanto o “gas producer train idler gearshaft” é levantado. Do the step (15). Remove the nut and washer from the “gas producer train idler spur gear” below the “gas producer train idler spur gearshaft”. Remove the Idler Spur Gear, bolts and washers from its support shaft while the gas producer train idler gearshaft is lifted. |  |  |
| 140 | Pag. 105 2.A | Efetuar etapas (16), (17). Remova o anel retentor interno da gaiola do rolamento do eixo. Remova o rolamento de esfera da tampa da gearbox. Remova o rolamento da carcaça da gearbox. Do the step (16) and (17). Remove the inner retaining ring from the shaft bearing cage. Remove the ball bearing from the gearbox cover. Remove the bearing from the gearbox housing. |  |  |
| 150 | Pag. 105 2.A | Efetuar etapas (18) e (19). Remova o anel de retenção do “idler spur gear” e separe o rolamento de esferas da engrenagem. Remova os três “idler gear support shafts” da tampa da gearbox. Do the steps (18) and (19). Remove the idler spur gear retaining ring and separate the ball bearing from the gear. Remove the three idler gear support shafts from the gearbox cover. |  |  |
| 160 | Pag. 105 2.A | Efetuar etapas (20) e (21). Remova os plug e packings do to da tampa da gearbox. Remova os selos. Do the step (20) and (21). Remove the plug and packing from the gearbox cover. Remove the seals. |  |  |
| 170 | Pag. 105 2.B | GEARBOX HOUSING DISASSEMBLY Efetuar passos (1) e (2). Remova o “fuel pump and oil pump spur gearshaft” e o “tachometer and governor power train spur gearshaft”. Do the steps (1) and (2). Remove the fuel control and oil pump spur gearshaft and the tachometer and governor power train spur gearshaft. |  |  |
| 180 | Pag. 106 2.B | Efetue o passo (3). Remova o “power train idler spur gear”. Do the step (3) Remove the power train idler spur gear. |  |  |
| 190 | Pag. 106 2.B | Efetuar passos (4) e (5). Suspenda a pista externa do rolamento e os roletes da “helical power takeoff gearshaft” da carcaça da gearbox. Remova a porca do “torquemeter shaft nut” Do the Steps (4) and (5). Lift the bearing outer race and the helical power takeoff gearshaft rollers from the gearbox housing. Remove the torquemeter shaft nut. |  |  |
| 200 | Pag. 107 2.B | Efetuar passos (6), (7) e (8). Remova o “torquemeter support shaft”, a placa final do rolamento e arruela, pistão e rolamento do eixo. Remova o expansor e o anel do pistão. Do the Steps (4) and (5). Remove the torquemeter support shaft, bearing end plate and washer, piston and shaft bearing. Remove the expander and piston ring. |  |  |
| 210 | Pag. 107 2.B | Efetuar passos (9) e (10). Remova o rolamento de esferas do pistão. Remova a pista externa do rolamento e roletes do support shaft. Remova a pista externa do rolamento do support shaft. Do the Steps (9) and (10). Remove the ball bearing from the piston. Remove the bearing outer race and support shaft rollers. Remove the outer bearing race from the support shaft. |  |  |
| 220 | Pag. 108 2.C | GEARBOX HOUSING LUBRICATION SYSTEM COMPONENTS DISASSEMBLY Efetuar passos (1) e (2). Se instalado, remova o indicador diferencial de pressão da tampa do filtro de óleo, remova a tampa e separe o filtro de óleo da carcaça de filtro. Suspenda a carcaça do filtro de óleo de lubrificação da carcaça da gearbox. Separe o tubo de pressão de óleo da carcaça da gearbox. Do the Steps (1) and (2). If installed, remove the differential pressure indicator from the oil filter cap, Remove the oil filter cap and eparate the oil filter and packings from the filter housing. lift the lube oil filter housing from the gearbox housing. Separate the oil pressure tube from the gearbox housing. |  |  |
| 230 | Pag. 108 2.C | Efetue os passos (3) e (4). Separe o tubo de entrega de óleo da carcaça da gearbox e remova os três tubos de transferência de óleo. Do the Steps (3) and (4). separate the oil delivery tube from the gearbox housing and remove the three oil transfer tubes. |  |  |
| 240 | Pag. 108 2.C | Efetue o passo (5). Remova a check valve. Do the step (5). Remove the check valve. |  |  |
| 250 | Pag. 108 2.C | Efetue o passo (6). Remova o fuel control e “oil pump flex shaft coupling”. Do the step (6). Remove the fuel control and oil pump flex shaft coupling. |  |  |
| 260 | Pag. 108 2.C | Efetue o passo (7). Remova a admissão do filtro de óleo e tubos de by-pass e os tubos de transferência da carcaça do filtro e o tubo de retorno de óleo. Remova a bomba. Do the step (7). Remove the oil filter inlet and bypass tubes, the transfer tube from the filter housing, and the scavenge oil tube. |  |  |
| 270 | Pag. 108 2.C | Efetue o passo (8). Remova os dois tubos de transferência de óleo e conector de baixo da bomba de óleo. Do the step (8). Remove the two oil transfer tubes and the connector beneath the oil pump. |  |  |
| 280 | Pag. 108 2.C | Efetue o passo (9). Remova o plug magnético da carcaça da gearbox. Do the step (9). Remove the magnetic plug from the gearbox housing |  |  |
| 290 | Pag. 108 2.C | Efetue o passo (10). Remova a check valve de retorno de baixa pressão da carcaça da gearbox. Do the step (10). Remove the lockwire and separate the magnetic plug from the gearbox housing. |  |  |
| 300 | Pag. 108 2.C | Efetue o passo (11). Remova os blocos de vedadores acessórios da carcaça da gearbox, apenas se os vedadores estiverem defeituosos. Do the step (11). Remove gearbox housing accessory pad seals, only if the seals are defective. |  |  |

NOTA: Não remova cages, studs, inserts ou pins mostrados na figura 4 (Ref. 72-60-00 - Description)

NOTE: Do not remove cages, studs, inserts or pins shown on figure 4 (Ref. 72-60-00 - Description)

| Observações: Observation: |  |  |  |
| --- | --- | --- | --- |
| Aprovação Approval | Nome Name | Data Date / / | Assinatura Signature |