| Artigo S/Nº: Item S/Nº: | Artigo P/Nº: Item P/Nº: | Manual: Manual: | Overhaul Manual 10W3 |
| --- | --- | --- | --- |
| Operador: Customer: | TLN: 02118 | Edição: Issue: | E4R16-72-2 |
|  |  | Data: Date: | June. 01/2020 |

| Operação Sequence | Referência Reference | Procedimentos / Tolerâncias Procedures / Serviceable Limits | Executado Executed by | Inspecionado Inspected by |
| --- | --- | --- | --- | --- |
| 10 | NOTE: | A ficha de desmontagem da turbina, leva em conta que já foi realizada a desmodulação, retirada de acessórios e parede de fogo, bem como a instalação na ferramenta P/N 6799955. The Turbine assembly disassembly sheet, considers that demodulation, accessories and firewall removal and turbine installation in tool P/N 6799955 has been already done. | ___/___/___ | ___/___/___ |
| 20 | OHM 72-50-00 Item (12) | Retire a “Spanner nut” do Rolamento 5 com auxílio das ferramentas P/N 6795984 e P/N 6795985. Remove the Ball bearing No. 5 Spanner nut using the holder and wrench P/N 6795984 and P/N 6795985. | ___/___/___ | ___/___/___ |
| 30 | OHM 72-50-00 Item (13) | Remova a trava e espaçador e então utilizando ferramenta P/N 6872882 (6795590) retire o Rolamento 5 da turbina. Retire a “Thrust plate”, elevando-a. Remove the retaining ring and washer and then, using the puller P/N 6872882 (6795590) remove the Ball bearing No. 5 from the turbine. Remove the thrust plate lifting it out. | ___/___/___ | ___/___/___ |
| 40 | OHM 72-50-00 Item (16) | Retire os parafusos e cuidadosamente, levante o “Exhaust”, separando-o do restante da turbina. Remove the screws and nuts and carefully lift the exhaust collector from the turbine assy. | ___/___/___ | ___/___/___ |
| 50 | OHM 72-50-00 Item (19) | Retire o “Bellows” da turbina com a ferramenta P/N 6872176. Remova a “Thrust plate” localizada abaixo do “bellows”. Pull the oil bellows from Turbine assy. With Seal Puller P/N 6872176. Remove thrust plate under the oil bellows. | ___/___/___ | ___/___/___ |
| 60 | OHM 72-50-00 Item (23) (24) | NOTA: Rotacione a turbina na ferramenta para trabalhar agora, na seção do rolamento 8. Com as ferramentas P/N 6795644 e 6795588 para retirar a “Spanner nut” do “Gas producer”. Utilize da fermenta P/N 6799955 para retirar o “oil sump nut”. NOTE: Rotate the turbine in the fixture to work now, in the Bearing No. 8 section. With the wrench fixtures P/N 6795644 and 6795588. Use the Fixture P/N 6799955 to remove the oil sump nut. | ___/___/___ | ___/___/___ |
| 70 | OHM 72-50-00 Item (26), (27) | Use a ferramenta P/N 6796920 para remover o injetor do rolamento 8 (Guarda-chuva) e retire o “Retaining plate”. Use the puller P/N 6796920 to remove the bearing No. 8 oil nozzle and remove the retaining plate. | ___/___/___ | ___/___/___ |
| 80 | OHM 72-50-00 Item (28) | Utilizando a ferramenta P/N 6795588 e um adaptador P/N 6795637 remova a “Spanner nut (LH)” do rolamento 8. Agora, com a ferramenta P/N 6795590, puxe o rolamento 8 e “Labirinth seal”. Using the wrench P/N 6795588 and the adapter P/N 6795637, remove the bearing No. 8 Spanner nut (LH). Now, with the puller P/N 6795590, pull the bearing No. 8 and the labyrinth seal. | ___/___/___ | ___/___/___ |

| Operação Sequence | Referência Reference | Procedimentos / Tolerâncias Procedures / Serviceable Limits | Montado Assembly by | Inspecionado Inspected by |
| --- | --- | --- | --- | --- |
| 90 | OHM 72-50-00 Item (21), (22) | NOTA: Gire a ferramenta de fixação novamente com a turbina de modo mais confortável para que se trabalhe na seção do rolamento 6. AVISO: Antes de retirar o rolamento 6, use uma lâmina fina para empurrar os roletes do rolamento na direção da pista externa para evitar que a ferramenta os danifique. Remova o “Retaining ring”. Use a ferramenta P/N 6795645 para remover o rolamento 6. Remova os 8 “Dampers” e levante o “spacer” para retirá-lo. NOTE: Turn the fixture with the turbine again, in a way it becomes comfortable for you to work on the Bearing No. 6 section. WARNING: Before removing the bearing No. 6, the rollers must be pushed back into the outer race to avoid the Puller to damage it. Remove the retaining ring. Use the puller P/N 6795645 to remove the bearing No. 6. Remove the 8 dampers and lift the spacer to remove it. | ___/___/___ | ___/___/___ |
| 100 | OHM 72-50-00 Item (23) | Com as ferramentas P/N 6795644 e 6795588 retire a “Spanner nut” do “Gas producer”. With the wrench fixtures P/N 6795644 and 6795588 remove the spanner nut from gas producer. | ___/___/___ | ___/___/___ |
| 110 | OHM 72-50-00 Item (30) | Retire os 24 parafusos que ligam a “GP support” e a “Power turbine support” e separe-os. Marque com uma vibropeen os furos e os “D bolts”” em sua sequência. Com a ferramenta P/N 6795646, separe a “Power turbine support” da “GP support”. Remove the 24 bolts at the gas producer support and power turbine support and separate them. Mark with vibropeen the holes and D bolts on its sequence. With pusher P/N 6795646, separate power turbine support from GP support. | ___/___/___ | ___/___/___ |
| 120 | OHM 72-50-00 Item (30) | Remova o Rolamento 7 com auxílio da ferramenta sacadora P/N 6795646. Remova os 8 “dampers”. NOTE: Marque com uma vibropeen o rolamento 7, no lado contrário do S/N para identificação. Remove the bearing 7 with help of puller P/N 6795646. Remove the 8 dampers. NOTE: Mark with vibropeen the bearing No. 7, on the contrary side of S/N for identification. | ___/___/___ | ___/___/___ |
| 130 | OHM 72-50-00 Item (32), (33), (34) | Remova o freno e os parafusos do “Oil sump cover” e retire-o da “Power turbine support”. Remove the lockwire and bolts from oil sump cover and remove it from power turbine support. | ___/___/___ | ___/___/___ |

| Observações: Observation: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Aprovação Approval |  | Nome Name | Data Date ______/______/________ | Assinatura Signature |