| NÚMERO ¦ NUMBER FS.0705 |  |  | O.S: |  |
| --- | --- | --- | --- | --- |
| IDENTIFICAÇÃO / IDENTIFICATION |  |  |  |  |
| TÍTULO | INSPEÇÃO DOS TERMOPARES M250 |  |  |  |
| TITLE | Thermocouple Inspection M250 |  |  |  |
| PUBLICAÇÃO ¦ PUBLICATION |  | REFERÊNCIA ¦ REFERENCE | LOCALIZAÇÃO | DATA ¦ DATE |
| Rolls–Royce 250–C20 SERIES OVERHAUL MANUAL |  | 77–20–01 | TLN02127 | TR E3R22-72-1 Aug. 12, 2022 |
| PART NUMBER: |  |  | SERIAL NUMBER: |  |

INFORMAÇÕES GERAIS / GENERAL INFORMATION

This Inspection Sheet provides for description of the temperature measurement system. The temperature measurement system consists of four chromel–alumel single junction thermocouples and an associated integral harness. The thermocouple harness is basically a part of the turbine assembly. (FIG. 1)

AVISOS/ ADVICES

- Verifique se o manual a ultima efetividade de páginas

Check manuals last effective work packages.

- Verifique a condição de segurança dos equipamentos de suporte

Check the safety condition of the support equipment.

- Verifique a disponibilidade de todo material requerido

Check the availability of all required materials.

- Verifique as condições de segurança do ambiente e dos operadores

Check environmental and personal safety conditions.

EQUIPAMENTO DE PROTEÇÃO INDIVIDUAL / PERSONAL PROTECTIVE EQUIPMENT

PLANEJAMENTO / PLANNING

● Limpe os termopares com um pedaço de tecido macio e seco.

Inspect the thermocouple

| ITEM | PROCEDIMENTOS /TOLERÂNCIAS | Condição | Executante / Data |
| --- | --- | --- | --- |
| Item | Procedures/Serviceable limits | Condition | Performed by / Date |

| 1) Visual–lead terminals distorted, corroded, breaking out, or not secure | None | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
| --- | --- | --- | --- |
| 2 Visual–insulation breaking out | 0.187 in. (4.75 mm) of original insulation | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
| 3 Visual–sleeving tears, frays, or looseness at clamp end | None | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
| 4 Visual–sleeving tears, frays, or looseness at clamp end | None | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
| 5 Visual–cable damage causing possible shorts | None | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
| 6 Sheath wear | 0.015 in (0.38 mm) max or 0.005 in. (0.13 mm) max over more than 90° of circumference. | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
| 7 Thermocouple tips cracked, eroded, or show evidence of overtemperature operation. (Use 10x magnification.) | 1. No cracks acceptable. 2. Erosion equaling maximum of 25% tip area reduction. 3. No melting or evidence of overtemperature. | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
| 8 Visually check thermocouple tip for break--out of insulation | Can lose up to 0.187 in. (4.75 mm) of original insulation. | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
| 9 Improper function of thermocouple | The room temperature resistance value must be 2,000 ohms minimum between terminal and probe flange or metal shielding. Measure resistance with a 1--1/2 volt ohmmeter (Simpson meter or equivalent). | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
| NOTE: If oven baking does not restore resistance value proceed as follows: - Connect the thermocouple harness terminals to a millivolt meter. Connect a 1--1/2 volt ohmmeter between one terminal and probe flange or metal shielding. Apply 1200 ± 50°F (649 ± 28°C) temperature to each thermocouple in succession (Jetcal or equivalent). - An acceptable harness will indicate an upscale reading on the millivolt meter and a gradual increase in resistance. If resistance decreases or indicates a dead short, replace the thermocouple harness. If resistance of each probe increases above 2,000 ohms during this hot check, the harness is acceptable. |  |  |  |

| ITEM | PROCEDIMENTOS /TOLERÂNCIAS | Condição | Executante / Data |
| --- | --- | --- | --- |
| Item | Procedures/Serviceable limits | Condition | Performed by / Date |

| 10 Continuity and polarity check | Heat each probe in turn to 1250 ± 50°F (677 ± 28°C). | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
| --- | --- | --- | --- |
| Causes for rejection: |  |  |  |

| ITEM | PROCEDIMENTOS /TOLERÂNCIAS | Condição | Executante / Data |
| --- | --- | --- | --- |
| Item | Procedures/Serviceable limits | Condition | Performed by / Date |

|  | No movement – indicates an open circuit. | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
| --- | --- | --- | --- |
|  | Lower than normal temperature – indicates a short circuit. | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
|  | Down–scale travel – indicates reversal of polarity | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |
|  | Fluctuating readings (resulting from a gentle bending of the lead) – indicates an intermittent open/short circuit. | Aprovado/Aproved Rejeitado/Rejected Relatório/Report |  |

| Aprovação Inspetoria | Nome |  | Data |  | Assinatura |  |
| --- | --- | --- | --- | --- | --- | --- |
| Inspector Approval | Name |  | Date |  | Signature |  |