| NÚMERO ¦ NUMBER | OS | MODELO Model | PN | SN |
| --- | --- | --- | --- | --- |
| FS.0734 |  | 250-C20R SERIES | 23073207 |  |

| IDENTIFICAÇÃO / IDENTIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| TÍTULO | FICHA DE REGISTROS DE TESTE DO COMPRESSOR BLEED VALVE |  |  |  |
| TITLE | Compressor Bleed Valve test sheet |  |  |  |
| PUBLICAÇÃO ¦ PUBLICATION |  | REFERÊNCIA ¦ REFERENCE | TLN | DATA ¦ DATE |
| OVERHAUL MANUAL |  | OHM GTP5232-3 ATA 75-10-05 | 02121 | TR E3R22-72-1 Aug. 12/ 2022 |
|  |  |  | CONJUNTO MAIOR ¦ MAIN PART |  |
|  |  |  | PN | SN |

§-11. TEST AND CALIBRATION A – Test and calibrate the bleed valve as follows: (See Table 7.)

| ITEM | PROCEDIMENTOS /TOLERÂNCIAS | Condição | Data | Executante |
| --- | --- | --- | --- | --- |
| Item | Procedures/Serviceable limits | Condition | Date | Performed by |
| 1 | Install bleed valve on bleed control gage fixture 6872546. (See Figure 6.) |  |  |  |
| 2 | Adjust ambient pressure (Pa) to 14.7 psia (101 kPa) and apply a compressor discharge pressure (Pc) of 85.5 psia (572 kPa) and adjust the inlet and exit orifice size relationship to obtain an interstage pressure (Pi) of 37.2-37.5 psia (256-259 kPa) with the valve at the fully closed position. Record Pi. The fully closed position is defined as the touch of the valve to the valve seat. | Pi: |  |  |
| 3 | Check remaining test points for Pi value specified. |  |  |  |
| 4 | After calibration, tighten nozzle (4) to 35-45 lb in. (3.9-5.1 N.m). Secure with lockwire. | Torque: |  |  |

| Table 7 CALIBRATION LIMITS (P/N 23073207) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Pa Psia | Pc Psia | Test Condition | Pi Limits Psia |  | Pi (Psia) |
|  |  |  | Min | Max |  |
| 14.7 | 85.5 | Reducing (Pi) until Valve is just full closed** | 36.8 | 37.6 |  |
| 14.7 | 51.5 | Increasing (Pi) until Valve is just full open* | 20.5 | 22.0 |  |
| 5.45 | 34.0 | Reducing (Pi) until Valve is just full closed** | 13.7 | 14.9 |  |
| 5.45 | 19.0 | Increasing (Pi) until Valve is just full open* | 7.2 | 8.5 |  |
| ** Full closed is defined as the touch of the valve to the valve seat. |  |  |  |  |  |
| * Fully open is defined as a minimum open distance of at least 0.434 in. (11.02 mm). |  |  |  |  |  |

§-11. TEST AND CALIBRATION B – Leakage Test

| ITEM | PROCEDIMENTOS /TOLERÂNCIAS | Condição | Data | Executante |
| --- | --- | --- | --- | --- |
| Item | Procedures/Serviceable limits | Condition | Date | Performed by |
| B | Apply 125 psig (862 kPag) of 21.1 + 5°C (70 + 9°F) air to the Pc port and 30 psig (207 kPag) of 21.1 + 5°C (70 ± 9°F) air to the valve mounting Pi. Total leakage of Pc and Pi shall not exceed 0.25 pound per minute. During this portion of the acceptance test a P/N 6852734 or equivalent gasket shall be used between the valve assembly and the test fixture mounting pad. | Total leakage: |  |  |
| C | On units meeting service testing and location inspection and quality requirements, it is recommended that information pertinent to the following areas be reviewed and evaluated prior to re-lease of the unit for reinstallation on the engine. |  |  |  |
| D | Apply 125 psig (862kPag) of 21.1 ± 5°C (70 ± 9°F) air to the Pc port and 30 psig (207kPag) of 21.1 ± 5°C (70 ± 9°F) air to the valve mounting Pi. Total leakage of Pc and Pi shall not exceed 0.30 pound per minute. | Total leakage: |  |  |

Observações/ Remarks

| Aprovação | Nome |  | Data |  | Assinatura |  |
| --- | --- | --- | --- | --- | --- | --- |
| Approval | Name |  | Date |  | Signature |  |