| Artigo S/Nº: Item S/Nº: | Artigo P/Nº: Item P/Nº: | Manual: Manual: | H/M MANUAL – CSP 22005 72-50-00-400-801 |
| --- | --- | --- | --- |
| Operador: Customer: | TLN: 02027 | Edição: Issue: | Rev. 9 |
|  |  | Data: Date: | Jun 15/2020 |

| Operação Sequence | Referência Reference | Procedimentos / Tolerâncias Procedures / Serviceable Limits | Balanceado Balanced by | Inspecionado Inspected by |
| --- | --- | --- | --- | --- |
| 1 | Pag. 1009 1.(G).(3) | (i). Balancear o gas producer turbine rotor (72-51-00-01-115) como descrito abaixo: Assemble and balance the gas producer turbine rotor assembly (72-51-00-01-115) as matched sets (Ref. FIG. 1018) as follows: 2 Balancear o gas producer turbine rotor com uma tolerância de 1,44 mg. m (Ref. FIG. 1018, AXIS H). Balance the gas producer turbine rotor with a tolerance of 1,44 mg. m. (Ref. Fig. 1018, Axis H). Inicial LH ( ) mg.m Inicial RH ( ) mg.m |  |  |
| 2 |  | (i). Balancear o gas producer turbine rotor (72-51-00-01-115) como descrito abaixo: Assemble and balance the gas producer turbine rotor assembly (72-51-00-01-115) as matched sets (Ref. FIG. 1018) as follows: 2 Balancear o gas producer turbine rotor com uma tolerância de 1,44 mg. m (Ref. FIG. 1018, AXIS H). Balance the gas producer turbine rotor with a tolerance of 1,44 mg. m. (Ref. Fig. 1018, Axis H). Final LH ( ) mg.m Final RH ( ) mg.m |  |  |
| NCC |  |  | Data Validade¦Expiration Date |  |
| BALANCEADORA SCHENK HORIZONTAL NCP: 000286 SCHENK HORIZONTAL BALANCER NCP: 000286 |  |  | / / |  |

| Observações: Observation: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Aprovação Approval |  | Nome Name | Data Date | Assinatura Signature |