| NÚMERO DE SÉRIE |  | NÚMERO DA PARTE |  |  |  | MANUAL | OHM PN 3037333 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SERIAL NUMBER |  | PART NUMBER |  |  |  | MANUAL |  |
| CLIENTE |  | TSN |  |  |  | REVISÃO | 41 |
| CUSTOMER |  | TSO |  |  |  | REVISION |  |
| TIPO DE TESTE |  | CSN |  |  |  | DATA | 25/JUL/2022 |
| TYPE OF TEST |  | CSO |  |  |  | DATE |  |
|  |  | TSHSI |  |  |  |  |  |

| OBSERVAÇÕES ANTES DO TESTE / REMARKS BEFORE TEST |  |  |
| --- | --- | --- |

| 1 | PREPARAÇÃO PARA TESTE / PREPARATION FOR TESTING |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  | RESULTADO / RESULT |
| 1.10 | Install engine on dynamometer test stand.See overhaul manual, PN 3037333, 72-00-00, TESTING 1, page 704, item 6(A)(1). |  |  |  | OK ¦ NOK |
| 1.2 | Install test equipment. See overhaul manual, PN 3037333, 72-00-00, TESTING 1, page 704, item 6(B)(1). |  |  |  | OK ¦ NOK |
| 1.3 | Prime oil system. See overhaul manual, PN 3037333, 72-00-00, TESTING 1, page 720, item 6(B)(2). |  |  |  | OK ¦ NOK |
| 1.4 | Install vibration pick-ups according the following torque:RGB 11 lbf.in. / Gas Generator Case 24 lbf.in. / Bevel Box Flange 11 lbf.in. See overhaul manual, PN 3037333, 72-00-00, TESTING 1, page 720, item 6(C). |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  | VERIFICADO POR / VERIFIED BY |  |  |
| ASSINATURA SIGNATURE |  | DATA DATE | ASSINATURA SIGNATURE | DATA DATE |  |

| 2 | PREPARAÇÃO PARA TESTE / PREPARATION FOR TESTING |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  | RESULTADO / RESULT |
| 2.1 | Rig Power Lever Angle (PLA). If necessary, check that PLA indication on DAS control display matches PLA shown on ARINC label 133/134 at 35 degrees. Check at 75 degrees and then recheck 35 degrees position. Difference shall not be greater than ± 1 degree.See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 711, item 7(B). |  |  |  | OK ¦ NOK |
| 2.2 | Rig Condition Lever Angle (CLA). See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 711, item 7(C). |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  | VERIFICADO POR / VERIFIED BY |  |  |
| ASSINATURA SIGNATURE |  | DATA DATE | ASSINATURA SIGNATURE | DATA DATE |  |

| 3 | PREPARAÇÃO PARA TESTE / PREPARATION FOR TESTING |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  | RESULTADO / RESULT |
| 3.1 | Depreservatethe engine. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 714, item 8(A). |  |  |  | OK ¦ NOK |
| 3.2 | During depreservation record the fuel flow. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 714, item 8(A)(5). Fuel Flow: ___________ pph (Fuel flow should not exceed 130 pph (58.98 Kg/h). |  |  |  | OK ¦ NOK |
| 3.3 | Perform wet motoring run. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 714, item 8(B). |  |  |  | OK ¦ NOK |
| 3.4 | During wet motoring run record the fuel flow. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 714, item 8(B)(4). Fuel Flow: ___________ pph (Fuel flow should not exceed 130 pph (58.98 Kg/h). |  |  |  | OK ¦ NOK |
| 3.5 | During wet motoring run verify fuel or oil leaks. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 714, item 8(B)(7). |  |  |  | OK ¦ NOK |
| 3.6 | During wet motoringcheck that there is no evidence of turbine rub. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 714, item 8(B)(8). |  |  |  | OK ¦ NOK |
| 3.7 | Perform dry motoring run. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 715, item 8(C). |  |  |  | OK ¦ NOK |
| 3.8 | Verify the oil system (Part 1). See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 716, item 9. |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  | VERIFICADO POR / VERIFIED BY |  |  |
| ASSINATURA SIGNATURE |  | DATA DATE | ASSINATURA SIGNATURE | DATA DATE |  |

| 4 | PARTIDA E CORTE DO MOTOR / ENGINE STARTING AND SHUTDOWN |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  |  |  |  |  |  | RESULTADO / RESULT |
| 4.1 | Perform the prestart checks. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 716, item 10(A). |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| 4.2 | Perform a general starting procedure. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 716, item 10(B). |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| 4.3 | Record the parameter bellow. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 716, item 10(B)(7). |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | NH |  | ________ rpm |  | NL | ________ rpm |  | NPT | ________ rpm |  |  |
|  | Q |  | _______ lbf.ft. |  | DELMOP | ________ psid |  | MOT | _________°F |  |  |
| 4.4 | Report any unsatisfactory start. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 716, item 10(C). |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| 4.5 | During engine shutdown check fuel dump valve and drains for correct operation.See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 719, item 10(D)(7). |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| 4.6 | During engine shutdown check fuel dump valve and drains for correct operation.See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 719, item 10(D)(8). |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  |  |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |  |  |
| ASSINATURA SIGNATURE |  |  |  | DATA DATE |  |  | ASSINATURA SIGNATURE |  |  |  | DATA DATE |

| 5 | VERIFICAÇÃO DO SISTEMA DE ÓLEO / VERIFICATION OF OIL SYSTEM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  |  |  |  | RESULTADO / RESULT |
| 5.1 | Perform oil pressure check. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 719, item 11(A). |  |  |  |  |  |  |  |  |
| 5.2 | Start engine and advance PLA to NH = 28300 rpm and an NPT of 20000 rpm. Runfor 3 minutes minimum.Check that MOT is 76.7 to 87.8°C (170-190°F). |  |  |  |  |  |  |  | OK ¦ NOK |
|  | NH | _______ rpm | NL | ________ rpm |  | NPT |  | _________ rpm |  |
|  | DELMOP | _______ psid | MOT | _______°F |  | Plt P8 |  | _______ psia |  |
| 5.3 | Decrease PLA to FI.Check that DELMOP is 57 to 63 psid. |  |  |  |  |  |  |  | OK ¦ NOK |
|  | NH | _______ rpm | NL | ________ rpm |  |  | NPT | _________ rpm |  |
|  | DELMOP | _______ psid | MOT | _______°F |  |  |  |  |  |
| 5.4 | Decrease PLA to GI. Check that DELMOP is 55 psid minimum. |  |  |  |  |  |  |  | OK ¦ NOK |
|  | NH | _______ rpm | NL | ________ rpm |  |  | NPT | _________ rpm |  |
|  | DELMOP | _______ psid | MOT | _______°F |  |  |  |  |  |
| 5.5 | Check oil level datum oil. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 719, item 11(B). Check and record the oil level and MOT accurately after 5 minutes from shutdown. |  |  |  |  |  |  |  | OK ¦ NOK |
|  | MOT | _______°F | Oil Level |  | __________________ |  |  |  |  |
| 5.6 | Perform functional check of oil pressure Check Valve. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 720, item 11(C)(1). Start engine and observe a sudden increase in DELMOP at NH 8325 to 13320rpm. Record NH rpm at point of DELMOP increase. |  |  |  |  |  |  |  | OK ¦ NOK |
|  | NH | _________________ rpm |  |  |  |  |  |  |  |
| 5.7 | Perform functional check of oil pressure check valve. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 720, item 11(C)(2). Shut engine down and observe a sudden drop of DELMOP in the NH range of12654 to 9324 rpm. Record NH rpm at point of DELMOP decrease. |  |  |  |  |  |  |  | OK ¦ NOK |
|  | NH | _________________ rpm |  |  |  |  |  |  |  |
| 5.8 | If DELMOP does not increase or decrease within specified range, do as overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 721, item 11(C)(3). |  |  |  |  |  |  |  | OK ¦ NOK |

| 5.9 | Perform functional check of oil pressure check valve. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 721, item 11(C)(4). Record any adjustment. |  |  |  |  |  | OK ¦ NOK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EXECUTADO POR / PERFORMED BY |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |  |
| ASSINATURA SIGNATURE |  |  | DATA DATE | ASSINATURA SIGNATURE | DATA DATE |  |  |

| 6 | CALIBRAÇÃO INICIAL / INITIAL CALIBRATION |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  |  |  |  |  | RESULTADO / RESULT |
| 6.1 | Engine test operating limits (refer to overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 705, table 701): |  |  |  |  |  |  |  |  |  |
|  | NH | rpm |  | SteadyState Transient |  |  |  | 34360 35440 |  |  |
|  | NL | rpm |  | SteadyState Transient |  |  |  | 28870 29575 |  |  |
|  | NPT | rpm |  | SteadyState Transient |  |  |  | 20500 22020 |  |  |
|  | T6 | °F |  | Start |  | Untrimmed (T6H) Trimmed (T6T) |  | 1802 1742 |  |  |
|  |  |  |  | Steady State |  | Untrimmed (T6H) Trimmed (T6T) |  | 1532 1472 |  |  |
|  |  |  |  | Transient |  | Untrimmed (T6H) Trimmed (T6T) |  | 1604 1544 |  |  |
|  | P3 | psia |  |  |  |  |  | 250 |  |  |
|  | Torque (Q) | lbf.ft. |  | SteadyState Transient |  |  |  | 12800 14440 |  |  |
| 6.2 | Start engine and run to the following dynamometer torque (Q). Stabilize for 4minutes at each setting and record a full set of readings. Process readings using program PWC83000. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 721, item 12. Power settings (SHP/: |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | 1) 2750 |  | 2) FI |  | 3) 1400 |  | 4) 2475 |  | 5) 2132 |  |

| 6.3 | Record any relevant event: |  |  |  |  |  | OK ¦ NOK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EXECUTADO POR / PERFORMED BY |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |  |
| ASSINATURA SIGNATURE |  |  | DATA DATE | ASSINATURA SIGNATURE | DATA DATE |  |  |

| 7 | VERIFICAÇÃO DO ESPECTRO DE VIBRAÇÃO / VIBRATION SPECTRUM CHECK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  |  |  |  |  |  |  | RESULTADO / RESULT |
| 7.1 | Perform vibration spectrum check. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 722, item 13. |  |  |  |  |  |  |  |  |  |  |  |
|  | Location |  | Information |  | Limits | Peak |  | RPM |  |  |  |  |
|  | Gas Generator |  | NH rotor Non-Synchronous Range: (30000 at 35000 rpm)NH |  | 0.3 | cm/secRMS |  | NH |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  | NPT |  |  |  |  |
|  |  |  |  |  |  |  |  | NL |  |  |  |  |
|  |  |  | 1 ENPT Range: (30000 at 35000 rpm)NH |  | 0.5 | cm/secRMS |  | NH |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  | NPT |  |  |  |  |
|  |  |  |  |  |  |  |  | NL |  |  |  |  |
|  |  |  | 1 ENH Range: (30000 at 35000 rpm)NH |  | 0.5 | cm/secRMS |  | NH |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  | NPT |  |  |  |  |
|  |  |  |  |  |  |  |  | NL |  |  |  |  |
|  |  |  | 1.02 ENH Range: (30000 at 35000 rpm)NH |  | 0.175 | cm/secRMS |  | NH |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  | NPT |  |  |  |  |
|  |  |  |  |  |  |  |  | NL |  |  |  |  |
|  |  |  | 1 ENL Range: (30000 at 35000 rpm)NH |  | 0.5 | cm/secRMS |  | NH |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  | NPT |  |  |  |  |
|  |  |  |  |  |  |  |  | NL |  |  |  |  |
|  |  |  | Overall |  | 4.0 | cm/secRMS |  | Take-off |  |  |  | OK ¦ NOK |
|  | Bevel Box |  | Range: (30000 at 35000 rpm)NH |  | 21.0 | m/sec2RMS |  | NH |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  | NPT |  |  |  |  |
|  |  |  |  |  |  |  |  | NL |  |  |  |  |
|  | RGB |  | Overall |  | 830 | m/sec2RMS |  | Take-off |  |  |  | OK ¦ NOK |
| 7.2 | Record any relevant event: |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  |  |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |  |  |  |
| ASSINATURA SIGNATURE |  |  |  | DATA DATE |  |  | ASSINATURA SIGNATURE |  |  | DATA DATE |  |  |

| 8 | DETERMINAÇÃO DO RESISTOR DE AJUSTE DE T6 / T6TRIM RESISTOR DETERMINATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  | RESULTADO / RESULT |
| 8.1 | Prior T6 trim determination, verify ITT/T6 according overhaul manual, PN 3037333, 72-00-00, TESTING 3, page 702, item 5. |  |  |  |  | OK ¦ NOK |
| 8.2 | Calculate T6T (target), see overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 723, item 14(A)(2). T6T (target) = ____________ °F |  |  |  |  | OK ¦ NOK |
| 8.3 | Adjust dial on trim panel to obtain T6T (target). See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 723, item 14(A)(4). |  |  |  |  | OK ¦ NOK |
| 8.4 | Measure and record resistance at test cell leads E and F. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 723, item 14(A)(7). Resistance = ____________ Ω |  |  |  |  | OK ¦ NOK |
| 8.5 | Select the appropriate T6 trim resistor. See overhaul manual, PN 3037333, 72-00-00, TESTING 3, page 702, item 5(F). |  |  |  |  | OK ¦ NOK |
|  | Resistance = ____________ Ω |  | PN: ___________________________ |  |  |  |
| EXECUTADO POR / PERFORMED BY |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |
| ASSINATURA SIGNATURE |  | DATA DATE |  | ASSINATURA SIGNATURE | DATA DATE |  |

| 9 | VERIFICAÇÃO DO CONECTOR DE CARACTETIZAÇÃO DO TORQUE DA EEC E AFU EEC AND AFU TORQUE CHARACTERIZATION TRIM PLUG CHECK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  |  | RESULTADO / RESULT |
| 9.1 | With original torquemeter trim plugs installed, check that Q-AFU, Q-EEC and Q are within 50 lbf.ft. of each other at 1400 and 2750 (Q) SHPpower settings. Seeoverhaul manual, PN 3037333, 72-00-00, TESTING 2, page 725, item 15(A). |  |  |  |  |  | OK ¦ NOK |
| 9.2 | Record any adjustment. Seeoverhaul manual, PN 3037333, 72-00-00, TESTING 2, page 725, item 15(B). |  |  |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |  |
| ASSINATURA SIGNATURE |  |  | DATA DATE | ASSINATURA SIGNATURE | DATA DATE |  |  |

| 10 | VERIFICAÇÃO DO CONTROLE – PARTE I / CONTROL CHECK – PART I |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  |  | RESULTADO / RESULT |
| 10.1 | Record EEC fault, if necessary. Seeoverhaul manual, PN 3037333, 72-00-00, TESTING 2, page 733, item 16(B). |  |  |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |  |
| ASSINATURA SIGNATURE |  |  | DATA DATE | ASSINATURA SIGNATURE | DATA DATE |  |  |

| 11 | VERIFICAÇÃO DE DESACELERAÇÃO E FLAME-OUT/DECELERATION/FLAME-OUT CHECK |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  | RESULTADO / RESULT |
| 11.1 | Perform check in solenoid manual mode OFF, according overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 749, item 17. No flame-out permitted. |  |  |  | OK ¦ NOK |
| 11.1 | Perform check in solenoid manual mode ON, according overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 749, item 17. No flame-out permitted. |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  | VERIFICADO POR / VERIFIED BY |  |  |
| ASSINATURA SIGNATURE |  | DATA DATE | ASSINATURA SIGNATURE | DATA DATE |  |

| 12 | VERIFICAÇÃO DO GOVERNADOR DE VELOCIDADE DE NH/NH GOVERNING CHECK |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  |  |  |  |  | RESULTADO / RESULT |
| 12.1 | Verify NH stability. If stable record NH, QTOT and TARGET TORQUE. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 749, item 18. |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | NH | ________ rpm |  | QTOT | _________ lbf.ft. |  | TARGET TORQUE | _________ lbf.ft. |  |  |
| EXECUTADO POR / PERFORMED BY |  |  |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |  |  |
| ASSINATURA SIGNATURE |  |  | DATA DATE |  |  | ASSINATURA SIGNATURE |  |  | DATA DATE |  |

| 13 | OPERAÇÃO DO MOTOR /ENGINE HANDLING |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  |  |  |  |  |  |  | RESULTADO / RESULT |
| 13.1 | Set de control panel. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 750, item 19(A). |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| 13.2 | Perform acceleration check (EEC Mode). See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 750, item 19(B). |  |  |  |  |  |  |  |  |  |  |  |
|  | - Advance power lever until T.O. and record: |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  |  | NH (rpm) | NL (rpm) |  | NPT (rpm) | Q (fbf.ft.) |  | T1 (° F) | PLA (°) |  |  |  |
|  | - Decrease power lever to 95% of T.O. Q recorded in paragraph (a)andrecord NH. NH: _______________ rpm |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Decrease power lever to FI and record: |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  |  | NH (rpm) | NL (rpm) |  | NPT (rpm) | Q (fbf.ft.) |  | T1 (° F) | PLA (°) |  |  |  |
|  | - Slam-accelerate from FI to 95% of T.O. Acceleration shall be timed to NH recorded in item (b). Retard power lever to GI. The acceleration time is not toexceed range specified in Figure 709. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 752. Time: _______________ seconds |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| 13.3 | Perform bodie checks. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 750, item 19(D). |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  |  |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |  |  |  |
| ASSINATURA SIGNATURE |  |  |  | DATA DATE |  |  | ASSINATURA SIGNATURE |  |  | DATA DATE |  |  |

| 14 | VERIFICAÇÃO DO NÍVEL DE ÓLEO /OIL LEVEL CHECK |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  |  |  |  |  |  | RESULTADO / RESULT |
| 14.1 | Perform oil level check. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 755, item 20. - Stop engine. - Wait 5 minutes after shutdown. - Record: |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  |  | Oil Level |  |  |  | MOT |  |  |  |  |  |
| EXECUTADO POR / PERFORMED BY |  |  |  |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |  |  |
| ASSINATURA SIGNATURE |  |  | DATA DATE |  |  |  | ASSINATURA SIGNATURE |  |  | DATA DATE |  |

| 15 | VERIFICAÇÃO DO CONTROLE – PARTE II / CONTROL CHECK – PART II |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RESULTADO / RESULT |
| 15.1 | Check NP fuel governing. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 754, item 21(B). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| 15.2 | Perform manual mode start. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 755, item 21(C). - With MANUAL SOLENOID - ON and start engine with power lever set to 20degrees. Ensure that NH accelerates smoothly and then stabilizes. - Verify NH stability does not exceed ± 50 rpm.Record NH, which must be between 24800 to 25500 rpm. NH ________________rpm. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - If necessary, adjust MFC. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 754, item 21(C)(5). MCF adjusted ¦ MCF not adjusted |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Advance power lever to 35° and record NH, which must be between 25640 to 26240 rpm. NH ________________rpm. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Advance power lever to obtain T.O. power with 2750 SHP/DELRTH and NPT20000 rpm and stabilize. Record: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  |  | NH (rpm) | NL (rpm) |  |  | NPT (rpm) |  | Q (fbf.ft.) |  |  | PLA (°) |  | Stability |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK NOK |  |  |  |
|  | - Retard power lever to GI and record NH, which must be between 24800 to 25550 rpm. NH ________________rpm. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| 15.3 | Perform manual mode start. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 757, item 21(D). - Select ATO on RATING switch. - SwitchCAN NPT GOV FOKKER - OFF. - Advance power lever to 75 degrees. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Record: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  |  | NH (rpm) | NL (rpm) |  |  | NPT (rpm) |  | QTOT |  |  | TARGET TORQUE |  | T6H |  |  |  |  |
|  | - Switch UPTRIM ON. Record: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  |  |  | QTOT |  |  |  |  | TARGET TORQUE |  |  |  |  |  |  |  |  |  |
|  | - Switch UPTRIM OFF (down) and verify that the engine returns to ATO ratingcondition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Decrease to GI. Record any fault codes displayed on ARINC label 240 if the ECS fault light is. Record: NH, NL, NPT, MOT and T1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  |  | NH (rpm) |  | NL (rpm) |  |  | NPT (rpm) |  | MOT |  |  | T1 |  |  |  |  |  |
|  | - Switch CAN NP GOV FOKKER - ON. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.4 | Perform EEC fail fixed check and overspeedgovernor check. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 758, item 21(E). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Remove mounting pad cover, install the overspeed governorpump on AGB without the Py tubeand speed reset solenoid cable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Advance power lever to 64 °, stabilize and record NH. The hydraulic pressure should be 1000 ± 50 psi. NH ________________rpm. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Select INHIBIT RESET - ON and verify FAIL FIXED lamp is on. Lamp ON ¦ Lamp OFF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Observe NH over a 20 to 30 second period. Verify that NH stabilizes and does not change by more than 400 rpm from NHrecorded in item (b), preceding. NH ________________rpm. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Reduce power lever to 25°. Verify that NH stays within range of 28000 rpmto RPM recorded in item (d), preceding. NH ________________rpm. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Select MANUAL SOLENOID to ON.Verify that a smooth decel transition occurs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Select INHIBIT RESET – OFF and MANUAL SOLENOID to OFF. - Shutdown the engine. Remove the overspeedgovernor and pump and install mounting padcover on AGB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| 15.5 | Perform autofeather system checks. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 758, item 21(F). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  |  |  |  |  |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |  |  |  |  |  |
| ASSINATURA SIGNATURE |  |  |  |  | DATA DATE |  |  |  |  | ASSINATURA SIGNATURE |  |  |  |  | DATA DATE |  |  |

| 16 | CALIBRAÇÃO FINAL / FINAL CALIBRATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RESULTADO / RESULT |
| 16.1 | Perform final calibration. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 759, item 22. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Run engine to following torque values at NP/RTH 20000 rpm with observed maximum of 20500 rpm. Stabilize for four minutes at each setting and save all parameters in test cell database. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  |  | 2750 SHP |  |  |  | 2475 SHP |  |  |  |  | 2132 SHP |  |  |  |  | FI |  |  |  |  |  |  |  |
|  | - Process data obtained in item (a) using program PWC83000. Plot normalized values of WF, T6T, NL, NH against QSHP on engineperformance graph. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 761,Figure 711. Draw appropriate lines. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - From plotted curve, enter at 2750 (Q) SHP, determine and record values for NH,NL, T6T and WF in corrected engine performance block. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  |  |  | NH |  |  |  |  | NL |  | T6T |  |  |  |  | WF |  |  |  |  |  |  |  |  |
|  | - Check the parameters determined in item (b) against required parameters. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 762, Table715. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Check the parameters determined in item (b) against the following supplementary acceptance limits: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - NH Steady state stability limits± 50 rpm. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - NL Steady state stability limits± 50 rpm. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - NPT Steady state stability limits± 100 rpm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Torque: ± ½ % of full scale torque reading. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Maximum vibration acceptance limitsSee overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 722, item 13. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Location |  |  | Information |  |  |  |  | Limits |  |  |  | Peak |  |  |  | RPM |  |  |  |  |  |  |
|  | Gas Generator |  |  | NH rotor Non-Synchronous Range: (30000 at 35000 rpm)NH |  |  |  |  | 0.3 |  |  |  | cm/secRMS |  |  |  | NH |  |  |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NPT |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NL |  |  |  |  |  |  |
|  |  |  |  | 1 ENPT Range: (30000 at 35000 rpm)NH |  |  |  |  | 0.5 |  |  |  | cm/secRMS |  |  |  | NH |  |  |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NPT |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NL |  |  |  |  |  |  |
|  |  |  |  | 1 ENH Range: (30000 at 35000 rpm)NH |  |  |  |  | 0.5 |  |  |  | cm/secRMS |  |  |  | NH |  |  |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NPT |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NL |  |  |  |  |  |  |
|  |  |  |  | 1.02 ENH Range: (30000 at 35000 rpm)NH |  |  |  |  | 0.175 |  |  |  | cm/secRMS |  |  |  | NH |  |  |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NPT |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NL |  |  |  |  |  |  |
|  |  |  |  | 1 ENL Range: (30000 at 35000 rpm)NH |  |  |  |  | 0.5 |  |  |  | cm/secRMS |  |  |  | NH |  |  |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NPT |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NL |  |  |  |  |  |  |
|  |  |  |  | Overall |  |  |  |  | 4.0 |  |  |  | cm/secRMS |  |  |  | Take-off |  |  |  |  |  | OK ¦ NOK |
|  | Bevel Box |  |  | Range: (30000 at 35000 rpm)NH |  |  |  |  | 21.0 |  |  |  | m/sec2RMS |  |  |  | NH |  |  |  |  |  | OK ¦ NOK |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NPT |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NL |  |  |  |  |  |  |
|  | RGB |  |  | Overall |  |  |  |  | 830 |  |  |  | m/sec2RMS |  |  |  | Take-off |  |  |  |  |  | OK ¦ NOK |
|  | - Maximum accessory gearbox static pressure shall not exceed 5.0 psig (34.47kPag) at T.O. power. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Reduction gearbox oil pressure shall not be less than 60 psig (413.7 kPag) atT.O. power. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Maximum oil consumption shall not exceed 0.5 pph (0.22 Kg/h). See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 765, item 23(A)(1)(b). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  | - Record the NH/and NL/ speed at2750 SHP/DELRTH on the engine reference data plate. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OK ¦ NOK |
|  |  |  |  |  | NH/ |  |  |  |  |  |  | NL/ |  |  |  |  |  |  |  |  |  |  |  |
| EXECUTADO POR / PERFORMED BY |  |  |  |  |  |  |  |  |  |  |  |  |  | VERIFICADO POR / VERIFIED BY |  |  |  |  |  |  |  |  |  |
| ASSINATURA SIGNATURE |  |  |  |  |  |  | DATA DATE |  |  |  |  |  |  | ASSINATURA SIGNATURE |  |  |  |  |  |  | DATA DATE |  |  |

| 17 | VERIFICAÇÃO FINAL / FINAL CHECKS |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  | RESULTADO / RESULT |
| 17.1 | Perform final checks. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 765, item 23. |  |  |  |  |
|  | - Inspect engine for external fuel and oil leaks. No leakage is permissible exceptat following locations: |  |  |  | OK ¦ NOK |
|  | - Fuel pump, less than 7 cc/h. |  |  |  | OK ¦ NOK |
|  | - Fuel nozzle Manifold, none permitted. |  |  |  | OK ¦ NOK |
|  | - Starter oil seal cavity, less than 3 cc/h. |  |  |  | OK ¦ NOK |
|  | - Propeller shaft oil seal cavity, less than 5 cc/h. |  |  |  | OK ¦ NOK |
|  | - All other seals, 3 cc/h. |  |  |  | OK ¦ NOK |
|  | - MFC: fuel coming from either drain without blue color. |  |  |  | OK ¦ NOK |
|  | - MFC: fuel leakage from ‘‘MFC front left drain’’ (fuel manifold regulatingvalve drain) not to be verified. |  |  |  | OK ¦ NOK |
|  | - Oil consumption, less than 0.5 pph (0.22 Kg/h). |  |  |  | OK ¦ NOK |
|  | - Not allowed contamination and/or metal particles in the following parts: |  |  |  | OK ¦ NOK |
|  | - Slave main oil pressure filters. |  |  |  | OK ¦ NOK |
|  | - Reduction gearbox scavenge oil filter. |  |  |  | OK ¦ NOK |
|  | - Main oil tankmagnetic chip detector. |  |  |  | OK ¦ NOK |
|  | - Reduction gearbox magnetic chip detector. |  |  |  | OK ¦ NOK |
|  | - Install main oil tank and reduction gearbox chip detectors and new main oilpressure and reduction gearbox scavenge oil filter elements. |  |  |  | OK ¦ NOK |
|  | - Record T6 thermocouple trim cable or trim resistor class on engine referencedata plate. |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  | VERIFICADO POR / VERIFIED BY |  |  |
| ASSINATURA SIGNATURE |  | DATA DATE | ASSINATURA SIGNATURE | DATA DATE |  |

| 18 | PRESERVAÇÃO / PRESERVATION |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  | RESULTADO / RESULT |
| 18.1 | Perform Fuel system preservation. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 766, item 24(A). |  |  |  | OK ¦ NOK |
|  | Perform Fuel system preservation. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 767, item 24(B). |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  | VERIFICADO POR / VERIFIED BY |  |  |
| ASSINATURA SIGNATURE |  | DATA DATE | ASSINATURA SIGNATURE | DATA DATE |  |

| 19 | REMOÇÃO DO BANCO DE PROVAS / REMOVAL FROM TEST STAND |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ATIVIDADE / ACTIVITY |  |  |  | RESULTADO / RESULT |
| 19.1 | Remove engine from test stand. See overhaul manual, PN 3037333, 72-00-00, TESTING 2, page 768, item 25. |  |  |  | OK ¦ NOK |
| EXECUTADO POR / PERFORMED BY |  |  | VERIFICADO POR / VERIFIED BY |  |  |
| ASSINATURA SIGNATURE |  | DATA DATE | ASSINATURA SIGNATURE | DATA DATE |  |

| OBSERVAÇÕES APÓS O TESTE / REMARKS AFTER TEST |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DATA DATE |  | INSTALAÇÃO / INSTALLATION |  |  | REMOÇÃO / REMOVAL |  |  |
| CONSUMO CONSUMPTION |  | COMBUSTÍVEL / FUEL |  |  | ÓLEO / OIL |  |  |
| COMBUSTÍVEL FUEL |  | TIPO / TYPE |  |  | LHV |  |  |
| ÓLEO OIL |  | NORMA / STANDARD |  |  | NOME / NAME |  |  |
| Certifico que o motor PW127M, número de série ________________ , foi testado e aprovado conforme manual de revisão geral, PN 3037333, revisão 36 de 30 de julho de 2018, ATA 72-00-00. O presente documento não atesta a aprovação para retorno ao serviço deste artigo. |  |  |  |  |  |  |  |
| I certify that the motor PW127M, serial number ________________, has been tested and approved in accordance with overhaul manual, PN 3037333, revision 36 of July 30, 2018, ATA 72-00-00. This document does not certify approval for return to service of this article. |  |  |  |  |  |  |  |
| INSPETOR DESIGNADO / DESIGNATED INSPECTOR |  |  |  |  |  |  |  |
| NOME NAME |  |  | REGISTRO REGISTRATION CODE | DATA DATE |  | ASSINATURA SIGNATURE |  |