T56-A-15 ENGINE DATA

| TURBINE | GEARBOX | COMPRESSOR | ENGINE | TORQUEMETER |
| --- | --- | --- | --- | --- |
| SN: | SN: | SN: | SN: | SN: |
| TYPE TEST: | TYPE TEST: | TYPE TEST: | TYPE TEST: | TYPE TEST: |

OVERHAUL MANUAL 15RC3 ATA 72-00-00-700-801 ENGINE TESTING

REVISION 14 – JUNE 30/2024

| ENGINE STATUS |  |
| --- | --- |

| PERFORMANCE DATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| PERFORMANCE POINT | TAKEOFF | MILITARY | NORMAL | 75% NORMAL |
| SPEC MGT (°F) | 1970 | 1920 | 1850 | 1610 |
| SPEC/PREDICTED SHP (HP) | 4643 / | 4419 / | 4111 / | 3046 / |
| SPEC /PREDICTED SFC (lb/hr/HP) | 0.529 / | 0.537 / | 0.549 / | 0.607 / |
| PERCENT DELTA FROM SPEC PWR (%) |  |  |  |  |
| PERCENT DELTA FROM SPEC SFC (%) |  |  |  |  |

| STARTS AND SHUTDOWN DATA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TIME START (HOURS) | DATE DD/MM/AA | MGT MAX °F | TIME SHUTDOWN (HOURS) | FUEL USED (LITERS) | RUN TIME (HOURS) HH:MM | REMARKS |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| : | / / | °F | : | L | : |  |
| TOTAL: |  |  |  | L | : |

PRE- START CHECK LIST: - Check air Pressure Test Cell; - Check dyno water valve A, B and C is open; - Check dyno oil lubrification is full; - Check all pressure and temperatures indication; - Check fuel valve is open; - Check vibration system is operating; - Check 28V and 115V power on; - Set overspeed trip; - Set overtemp trip; - Set overtorque trip; - Set Dyno PID values; - Power lever in ground; - Condition lever in cut-off; - Check dyno system is operating in SPEED MODE (1020 RPM IN OUTPUT SHAFT). NOTE: DO NOT START THE ENGINE IF THE TIT IS MORE THAN 392°F (200°C).

| COMPONENTS REPLACEMENT RECORD |  |  |  |
| --- | --- | --- | --- |
| ITEM | 1 | 2 | 3 |
| DESCRIPTION |  |  |  |
| PN (REMOVED) |  |  |  |
| SN (REMOVED) |  |  |  |
| SN ENG. (REMOVED) |  |  |  |
| PN (INSTALLED) |  |  |  |
| SN (INSTALLED) |  |  |  |
| SN ENG. (INSTALLED) |  |  |  |
| DATE | / / | / / | / / |
| APPROVED BY |  |  |  |

| ITEM | TEST PROCEDURE | REALIZED | APROVED |
| --- | --- | --- | --- |
| 1 | Install the dressing kit of the engine. |  |  |
| 2 | Make the following service connections to reduction gear and power unit: Fuel supply to power unit inlet Oil supply to reduction gear inlet Oil return from reduction gear outlet Oil supply to power unit inlet Oil return from power unit outlet Starter air supply to starter |  |  |
| 3 | Make the following pressure sensing connections: Power unit fuel manifold pressure Fuel pump discharge pressure Fuel inlet pressure. Oil inlet pressure Scavenge oil pressure Power unit oil pressure Reduction gear oil pressure Compressor air inlet pressure Test cell reference pressure Engine anti-icing air pressure Diffuser sump (lighthouse) pressure Turbine rear bearing oil pressure |  |  |
| 4 | Make the temperature measuring connections: CIT TIT P.S oil out temperature P.S oil in temperature R.G oil out temperature R.G oil in temperature |  |  |
| 5 | Reset Fuel Flow Meters: |  |  |
| 6 | Select engine model T56 in Texcel VX 100. |  |  |
|  | Check overspeed configuration. |  |  |
| 7 | Engine installed and checked. |  |  |
| 8 | Perform PLA and CLA rigging: |  |  |
| 9 | Perform the electrical components checkout (Test Set Tool PN : 6799053): 2200 RPM Switch: A 13000 RPM Switch: A Fuel Manifold Press. Switch: A Anti-Ice Solenoid Switch: A Ignition Exciter: A Ignition Relay and 9000 RPM Switch: A Fuel Enrichment Solenoid: A Drip Valve Solenoid: A 20% Take Solenoid: A Ground Start Solenoid: A Pump Paralleling Valve: A Secondary pump Press. Switch: A Fuel Shutoff Actuator (Genevalock): Power Lever Switches: |  |  |
| 10 | Perform the Temperature Datum System Testing (Tool PN: 6798233) |  |  |
| 11 | Perform pre-start check list (page 2). NOTE 1: DO NOT START THE ENGINE IF THE MGT IS MORE THAN 392°F (200°C) NOTE 2: MAKE SURE THE AIR PRESSURE IS MORE THAN 30 PSIG |  |  |
| 12 | Depreserve fuel system. |  |  |
| 13 | During the last motoring there was positive Oil Pressure? |  |  |
| 14 | Start Engine. NOTE 1: PERFORM PRE-START LIST BEFORE START. NOTE 2: ALL STARTS MUST BE RECORDED IN WORK INSTRUCTION PAGE 2/9. NOTE 3: MAKE SURE THE AIR PRESSURE IS MORE THAN 30 PSIG |  |  |
| 15 | Records during start and stabilization in Ground Idle: T.T.I:. PSOP:. RGOP:. |  |  |
| 16 | Perform Seal Break-In if applicable: Engine is free of any abnormal noise and rubbing? Stop Time: sec |  |  |
| 17 | Low Speed Idle vibration is within the limits? Compressor: Max. 2 mils Gearbox: Max. 3 mils Turbine: Max. 4 mils |  |  |
| 18 | Perform on-site inspection of fuel, oil, and air leakage. Air leakage Oil leakage Fuel leakage |  |  |
| 19 | Perform Fuel control governor check: Fuel Flow: pph Engine RPM: rpm |  |  |
| 20 | Perform Brake Lock check: |  |  |
| 21 | Perform TD System check: Engine TIT: °F Datum TIT: °F |  |  |
| 22 | Rich-Lean check: TD in AUTO: TIT °F , PLA ° TD IN NULL: TIT °F , PLA ° The Fuel control difference is richer than not more 180°F or lean than not less 126°F? |  |  |
| 23 | During the take-off stabilization check: P.S Oil Press.; Min. 50 psig; Max. 60 psig; psig. R.G Oil Press.; Min. 170 psig; Max. 190 psig; psig. Turbine Vib.; Máximum 2 mils; mils. Power Section Vib.; Maximum 2 mils; mils. RGB Vib.; Maximum 3 mils; mils. Speed Fluctuations; ± 70 rpm; rpm. |  |  |
| 24 | Anti-icing air operational check: anti-icing air valve gage: psig (min. 30 psig) |  |  |
| 25 | Reverse thrust operational check: Engine speed fluctuation: rpm (max. 100 rpm) TIT: °F (max. 1530°F) Engine Speed: rpm (limits in table 1318) |  |  |
| 26 | Torquemeter indicator calibration check (CAL A, CAL B): Torquemeter CAL NUMBER: CAL A: CAL B: difference between the indicated and calculated torque: LB-IN |  |  |
| 27 | Acceptable Run Schedule (Table 1319): Setting n° 1 – START ENGINE. ACCELERATE TO HIGH RPM. Setting n° 2 – 75% Normal. Setting n° 3 – Normal. Setting n° 4 – Military. Setting n° 5 – Takeoff. Setting n° 6 – Flight Idle. Setting n° 7 – ACCELERATE FROM FLIGHT IDLE TO TAKEOFF Setting n° 8 – TAKEOFF Setting n° 9 – DECELERATE TO FLIGHT IDLE Setting n° 10 – Flight Idle Setting n° 11 – Shutdown. Check operation of prop. brake and front seal for leakage. Setting n° 12 – START ENGINE. T.T.L: T.T.I: Setting n° 13 – Low-RPM Idle: RPM Setting n° 14 – High-RPM Idle: RPM |  |  |
| 28 | Bleed Air Contamination Check: 75% normal: Military: Takeoff: |  |  |
| 29 | Engine Shutdown. Engine is free of any abnormal noise and rubbing? Stop Time: sec |  |  |
| 30 | Perform Fuel system Preservation. |  |  |
| 31 | Final Inspections after shutdown: Chip detectors Inspection: Compressor Blades 5th and 10th stages: Rear Bearing Support cracks inspection: Accessories drain leaks: |  |  |
| 32 | Remove dressing kit and cap off. |  |  |
| 33 | Send engine and documentation for workshop. Write test cell engine test sheet. |  |  |

| REMARKS: |  |  |  |
| --- | --- | --- | --- |
| TEST CELL OPERATOR: | DATE: | TEST CELL INSPECTOR: | DATE: |
|  | / / |  | / / |