| NÚMERO ¦ NUMBER | OS | MODELO Model | PN | SN |
| --- | --- | --- | --- | --- |
| FS.0741 |  | DC STARTER GENERATOR | 23048SERIES |  |

| IDENTIFICAÇÃO / IDENTIFICATION |  |
| --- | --- |
| TÍTULO | FICHA DE REGISTRO DE INSPEÇÃO |
| TITLE | INSPECTION RECORD SHEET |

| PUBLICAÇÃO ¦ PUBLICATION | REFERÊNCIA ¦ REFERENCE | TLN | DATA ¦ DATE |
| --- | --- | --- | --- |
| COMPONENT MAINTENANCE MANUAL WITH IPL | ATA 24-30-74 | TLN02209 | REVISION 22 DATED OCTOBER, 30/2019 |
|  |  | CONJUNTO MAIOR ¦ MAIN PART |  |
|  |  | PN | SN |

TAREFAS / TASKS

| Item | Procedimentos | Critério | Resultado | Condição | Realizado por: |
| --- | --- | --- | --- | --- | --- |
| Item | Procedures | Criteria | Result | Condition | Performed by |
| §-8 A.(1) | VISUAL Identification Plate (345) | According Paragraph 3.A |  |  |  |
| §-8 A.(2) | VISUAL Identification Plate (345) | Make sure that the drive screws (350) are tight and in place |  |  |  |
| §-8 B.(1) | VISUAL Fan Covers (25) or Fan Cover Assy (25A) and Brush Cover (50) | According Paragraph 3.A |  |  |  |
| §-8 B.(2) | VISUAL Fan Covers (25) or Fan Cover Assy (25A) and Brush Cover (50) | Discard the part if cracks, major damage or fretting corrosion on either the inlet or mounting flange exists |  |  |  |
| §-8 B.(3) | VISUAL Fan Covers (25) or Fan Cover Assy (25A) and Brush Cover (50) | Repair the part if minor surface damage exists |  |  |  |
| §-8 B.(4) | VISUAL Fan Covers (25) or Fan Cover Assy (25A) and Brush Cover (50) | (Models 23048-016, -018 and -023 only) Make sure that gasket (30) in fan cover assy (25A) is not damaged or missing. Replace gasket if necessary. |  |  |  |
| §-8 C.(1) FIG. 5002 | VISUAL Brush Cover (50) | According Paragraph 3.A |  |  |  |
| Item | Procedimentos | Critério | Resultado | Condição | Realizado por: |
| Item | Procedures | Criteria | Result | Condition | Performed by |
| §-8 C.(2) FIG. 5002 | VISUAL Brush Cover (50) | Examine insulation tape for tears or loose edges |  |  |  |
| §-8 C.(3) | VISUAL Brush Cover (50) | Examine the screw (55) threads IAW the procedures found in Paragraph 3.A |  |  |  |
| §-8 C.(4) | VISUAL Brush Cover (50) | Make sure that the blind rivet nut (56) on the bracket is tightly attached |  |  |  |
| §-8 C.(5) | VISUAL Brush Cover (50) | Examine the blind rivet nut (56) threads IAW the procedures found in Paragraph 3.A |  |  |  |
| §-8 D.(1) | VISUAL Brushes (60) and Brush Holder Assy (220) | Visually examine brush holders for cracks, warpage and broken brush springs |  |  |  |
| §-8 D.(2) | VISUAL Brushes (60) and Brush Holder Assy (220) | Visually examine brushes for cracks, chips, frayed leads, and loose rivets or loose connections. Discard brushes if damaged. |  |  |  |
| §-8 D.(3) | VISUAL Brushes (60) and Brush Holder Assy (220) | Replace brushes if remaining allowable wear will be achieved before the next overhaul (See figure 5003). Seat new brushes in accordance with instructions in SPD 1006. |  |  |  |
| §-8 D.(4) Fig. 5004 | VISUAL Brushes (60) and Brush Holder Assy (220) | Measure brush spring (245) pressure using a pull scale with harness |  |  |  |
| §-8 E.(1) | VISUAL Air Inlet Assy (5) | Visually examine all surfaces for dents, nicks and scratches that reduce clearance between the sidewall and fan cover (25). |  |  |  |
| §-8 E.(2) | VISUAL Air Inlet Assy (5) | Visually examine mounting and inlet flange for gouging, scoring or glazing |  |  |  |

| Item | Procedimentos | Critério | Resultado | Condição | Realizado por: |
| --- | --- | --- | --- | --- | --- |
| Item | Procedures | Criteria | Result | Condition | Performed by |
| §-8 F.(1) Fig. 5005 | VISUAL Fan (70) | According Paragraph 3.A NOTE: Pay particular attention to the blade edges and surfaces and shaft mating surface. |  |  |  |
| §-8 F.(2) | VISUAL Fan (70) | If the visual inspection finds indications which can be cracks, continue the inspections under 10X magnification |  |  |  |
| §-8 F.(3) | VISUAL Fan (70) | If the visual inspection under 10X magnification finds indications which can be cracks, perform a FPI inspection |  |  |  |
| §-8 G.(1) Fig. 5006 | INSPEÇÃO DIMENSIONAL Two-part fan 23046-1020 | Dimensionally inspect the fan assembly according fig. 5006: a) Back edge b) Forward edge | VALOR NCC |  |  |
| §-8 H.(1) Fig. 5007 | INSPEÇÃO DIMENSIONAL Two-part fan 23048-1240 | Dimensionally inspect the fan assembly according fig. 5007: a) Back edge b) Forward edge | VALOR NCC |  |  |
| §-8 I.(1) | VISUAL Drive shaft (85) and Dampener Hub (100) | Visually examine all surface for cracks |  |  |  |
| §-8 I.(2) | VISUAL Drive shaft (85) and Dampener Hub (100) | Visually examine all surface for dents, nicks and scratches |  |  |  |
| §-8 I.(3) Fig. 5008 | VISUAL Drive shaft (85) and Dampener Hub (100) | Visually examine drive spline and dampener hub for hounding, stripping or uneven wear. Fig. 5008 |  |  |  |
| §-8 I.(4) | VISUAL Drive shaft (85) and Dampener Hub (100) | Visually examine screw threads for crossed or stripped threads |  |  |  |

| Item | Procedimentos | Critério | Resultado | Condição | Realizado por: |
| --- | --- | --- | --- | --- | --- |
| Item | Procedures | Criteria | Result | Condition | Performed by |
| §-8 I.(5) | VISUAL Drive shaft (85) and Dampener Hub (100) | Measure drive spline diameter over two gauge pins. |  |  |  |
| §-8 I.(6) | VISUAL Drive shaft (85) and Dampener Hub (100) | Perform a magnetic particle inspection IAW procedures found in paragraph 5.A | OS LAUDO |  |  |
| §-8 J.(1) | VISUAL Friction Ring (105) | If friction ring is part No. 02-5600-05, it must be discarded because it contains asbestos |  |  |  |
| WARNING: DO NOT CUT, RIP, OR SAND ASBESTOS-CONTAINING MATERIALS. LEAVE UNDAMAGED MATERIALS ALONE AND, TO EXTENT POSSIBLE, PREVENT THEM FROM BEING DAMAGED, DISTURBED OR TOUCHED. DISCARD MATERIAL BY FIRST CHECKING WITH LOCAL HEALTH, ENVIRONMENTAL, OR OTHER APPROPRIATE OFFICIALS TO FIND ABOUT PROPER HANDLING AND DISPOSAL PROCEDURES |  |  |  |  |  |
| §-8 J.(2) | VISUAL Friction Ring (105) | Visually examine all surfaces for cracks |  |  |  |
| §-8 J.(3) Fig. 5009 | INSPEÇÃO DIMENSIONAL Friction Ring (105) | Measure friction ring a) Thickness | VALOR NCC |  |  |
| §-8 K.(1) Fig. 5010 | VISUAL Dampener Plate (95) | See figure 5010. Visually examine all surfaces for cracks or warpage |  |  |  |
| §-8 K.(2) | VISUAL Dampener Plate (95) | Visually examine all surfaces for dents, nicks and scratches |  |  |  |
| §-8 K.(3) | VISUAL Dampener Plate (95) | Visually examine mating surfaces for gouging, scoring or glazing |  |  |  |
| §-8 K.(4) | VISUAL Dampener Plate (95) | Visually examine spline teeth for rounding, stripping or uneven wear |  |  |  |
| §-8 K.(5) | VISUAL Dampener Plate (95) | Measure dampener plate thickness |  |  |  |
| §-8 K.(6) | VISUAL Dampener Plate (95) | Measure the distance between the pins of the spline teeth |  |  |  |
| §-8 K.(7) | ENSAIO Dampener Plate (95) | Perform a MPI inspection according paragraph 5.A | OS LAUDO |  |  |
| §-8 L.(1) Fig. 5011 | VISUAL Dampener Back Plate (110) | See figure 5011. Visually examine all surfaces for cracks |  |  |  |
| Item | Procedimentos | Critério | Resultado | Condição | Realizado por: |
| Item | Procedures | Criteria | Result | Condition | Performed by |
| §-8 L.(2) | VISUAL Dampener Back Plate (110) | Visually examine all surfaces for dents, nicks and scratches |  |  |  |
| §-8 L.(3) | VISUAL Dampener Back Plate (110) | Visually examine mating surfaces for gouging, scoring or glazing |  |  |  |
| §-8 L.(4) | ENSAIO Dampener Back Plate (110) | Perform a MPI inspection according paragraph 5.A | OS LAUDO |  |  |
| §-8 M.(1) | VISUAL Drive End Bearing Support Assy (115) | According Paragraph 3.A |  |  |  |
| §-8 M.(2) | VISUAL Drive End Bearing Support Assy (115) | If the visual inspection finds indications which can be cracks, continue the inspections under 10X magnification |  |  |  |
| §-8 M.(3) | VISUAL Drive End Bearing Support Assy (115) | If the visual inspection under 10X magnification finds indications which can be cracks, perform a FPI inspection |  |  |  |
| §-8 M.(4) | INSPEÇÃO DIMENSIONAL Drive End Bearing Support Assy (115) | Measure the bearing liner diameter using a gage and equipment with an accuracy of ± 0.00004”. See fig. 5012 | VALOR NCC |  |  |
| §-8 N.(1) | VISUAL Baffle Discs (165) | Visually examine all surfaces for dents, nicks and scratches |  |  |  |
| §-8 N.(2) | VISUAL Baffle Discs (165) | Visually examine mating surfaces for gouging, scoring or glazing |  |  |  |
| CAUTION: INSULATION ENAMEL (GLYPTAL OR EQUIVALENT) IS NOT APPROVED ON THE ARMATURE WINDINGS. THE ARMATURE MUST BE REPLACED IF YOU FIND INSULATING ENAMEL ON THE ARMATURE WINDINGS |  |  |  |  |  |
| §-8 O.(1) | VISUAL Armature (175) | Before the commutator is refinished, measure commutator bar-to-bar run-out in a full circumference outside the brush paths (area where the brushes do not touch) |  |  |  |
| Item | Procedimentos | Critério | Resultado | Condição | Realizado por: |
| Item | Procedures | Criteria | Result | Condition | Performed by |
| §-8 O.(2) | VISUAL Armature (175) | According Paragraph 3.A |  |  |  |
| §-8 O.(3) | VISUAL Armature (175) | Examine bearing journals for gouging, scoring, or glazing |  |  |  |
| §-8 O.(4) Fig. 5013 | INSPEÇÃO DIMENSIONAL Armature (175) | Measure bearing journal diameters, A and C, fig. 5013 | VALOR NCC |  |  |
| §-8 O.(5) | VISUAL Armature (175) | Measure commutator (before recut) diameter “B”. fig 5013 |  |  |  |
| §-8 O.(6) | VISUAL Armature (175) | Examine internal spline for rounding, stripping, or uneven wear |  |  |  |
| §-8 O.(7) | VISUAL Armature (175) | Examine commutator undercut for broken, damaged or missing mica |  |  |  |
| §-8 O.(8) | VISUAL Armature (175) | Examine commutator bars for burning and discoloration |  |  |  |
| §-8 O.(9) | VISUAL Armature (175) | Measure commutator undercut |  |  |  |
| §-8 O.(10) | VISUAL Armature (175) | After commutator is refinished, measure commutator bar-to-bar and total indicator reading (TIR) run-out in a full circumference. Support the armature (175) on two “V” blocks |  |  |  |
| §-8 O.(11) | VISUAL Armature (175) | Test armature windings for shorts using a growler and iron feeler. |  |  |  |
| §-8 O.(12) | VISUAL Armature (175) | Perform a dielectric test |  |  |  |
| §-8 P.(1) | VISUAL Stator and Housing Assy (275) | Visually inspect exposed housing surfaces for cracks, chips and dents |  |  |  |

| Item | Procedimentos | Critério | Resultado | Condição | Realizado por: |
| --- | --- | --- | --- | --- | --- |
| Item | Procedures | Criteria | Result | Condition | Performed by |
| §-8 P.(2) | VISUAL Stator and Housing Assy (275) | Visually inspect for the following indications of overheating: . varnish coating of exposed windings is cracked or flaking . missing, melted or damaged insulators |  |  |  |
| §-8 P.(3) | VISUAL Stator and Housing Assy (275) | Check for tight fit of pole shoes and damage to windings in housing |  |  |  |
| §-8 P.(4) | VISUAL Stator and Housing Assy (275) | Perform a dielectric test |  |  |  |
| §-8 P.(5) | VISUAL Stator and Housing Assy (275) | Use an ohmmeter to check for continuity between terminals A and E |  |  |  |
| §-8 Q.(1) | VISUAL Terminal Block (280) | Visually examine all surfaces for dents, nicks and scratches |  |  |  |
| §-8 Q.(2) | VISUAL Terminal Block (280) | Visually examine terminal studs for crossed or stripped threads |  |  |  |
| §-8 Q.(3) | VISUAL Terminal Block (280) | Using a LCR (inductance, capacitance and resistance) meter, measure the capacitance values between terminals B and E, between B and ground and between C and ground. |  |  |  |
| §-8 R.(1) Fig. 5015 Fig. 5016 | VISUAL Bearing and Brush support assy ( 200) | According Paragraph 3.A |  |  |  |
| §-8 R.(2) | VISUAL Bearing and Brush support assy ( 200) | If the visual inspection finds indications which can be cracks, continue the inspections under 10X magnification |  |  |  |

| Item | Procedimentos | Critério | Resultado | Condição | Realizado por: |
| --- | --- | --- | --- | --- | --- |
| Item | Procedures | Criteria | Result | Condition | Performed by |
| §-8 R.(3) | VISUAL Bearing and Brush support assy ( 200) | If the visual inspection under 10X magnification finds indications which can be cracks, perform a FPI inspection |  |  |  |
| §-8 R.(4) | INSPEÇÃO DIMENSIONAL Bearing and Brush support assy (200) | Measure the bearing liner diameter “A” using a gage and equipment with an accuracy of ± 0.00004”. See fig. 5015 | VALOR NCC |  |  |
| §-8 R.(5) | VISUAL Bearing and Brush support assy ( 200) | Examine the heli-coil inserts for damage |  |  |  |
| §-8 R.(6) | VISUAL Bearing and Brush support assy ( 200) | Hand tighten a machine screw into the blind rivet nut in each complete brush holder (220) to assure that the self-locking feature is functional. The machine screw should bind in the blind rivet nut before it is fully engaged |  |  |  |
| §-8 R.(7) | VISUAL Bearing and Brush support assy ( 200) | Examine the brush holders (255), including the brush spring supports and center supports for cracks, warping or discoloration, caused by electrical arcing |  |  |  |
| §-8 R.(8) | VISUAL Bearing and Brush support assy ( 200) | Perform a dielectric test |  |  |  |
| §-8 S.(1) | VISUAL Bearing Retainer (180) | Visually inspect bearing retainer for cracks, chips and dents |  |  |  |
| §-8 S.(2) | VISUAL Bearing Retainer (180) | Place retainer on surface plate to check for warping. |  |  |  |
| §-8 T.(1) | VISUAL Drive End Pad End Bell (365) | Measure the height of the three locating dowels |  |  |  |

| Item | Procedimentos | Critério | Resultado | Condição | Realizado por: |
| --- | --- | --- | --- | --- | --- |
| Item | Procedures | Criteria | Result | Condition | Performed by |
| Table 8002 §8.D.(4).(c) | INSPEÇÃO DIMENSIONAL Brush Holder Assy (220) | Brush Spring Tension Position “A”: 2.6 lbs (11.1 N) min. | VALOR NCC |  |  |
| Table 8002 §8.D.(4).(c) | INSPEÇÃO DIMENSIONAL Brush Holder Assy (220) | Brush Spring Tension Position “B”: 3.4 lbs (15.3 N) min. | VALOR NCC |  |  |
| Table 8002 §8.I.(5) | INSPEÇÃO DIMENSIONAL Drive Shaft – 12 tooth (85) used on all models, but model 23048-023 | Drive End Spline dia. over two 0.1094” dia. pins 0.757” minimum | VALOR NCC |  |  |
| Table 8002 §8.I.(5) | INSPEÇÃO DIMENSIONAL Drive Shaft – 16 tooth (85) used on model 23048-023 | Drive End Spline dia. over two 0.096” dia. pins 0.921” minimum | VALOR NCC |  |  |
| Table 8002 §8.J | INSPEÇÃO DIMENSIONAL Friction Ring (105) | Thickness 0.060” minimum | VALOR NCC |  |  |
| Table 8002 §8.K | INSPEÇÃO DIMENSIONAL Dampener Plate (95) | Thickness 0.038” minimum | VALOR NCC |  |  |
| Table 8002 §8.K | INSPEÇÃO DIMENSIONAL Dampener Plate (95) | Internal spline dia. Over two 0.090” dia. pins 0.6550” maximum | VALOR NCC |  |  |
| Table 8002 §8.M.(4). (a) | INSPEÇÃO DIMENSIONAL Drive end bearing support (115) | Bearing Liner diameter 1.8501” – 1.8504” | VALOR NCC |  |  |
| Table 8002 §8.O | INSPEÇÃO DIMENSIONAL Armature (175) | Bearing Journal Diameter (A, C) 0.7872”-0.7875” | VALOR NCC |  |  |
| Table 8002 §8.O | INSPEÇÃO DIMENSIONAL Armature (175) | Commutator Diam. (B) 2.060” Min. | VALOR NCC |  |  |
| Table 8002 §8.O | INSPEÇÃO DIMENSIONAL Armature (175) | Commutator Bar to Bar Acceptance Test Values Bar to bar: 0.0002”Max TIR: 0.0008” Max | VALOR NCC |  |  |

| Item | Procedimentos | Critério | Resultado | Condição | Realizado por: |
| --- | --- | --- | --- | --- | --- |
| Item | Procedures | Criteria | Result | Condition | Performed by |
| Table 8002 §8.O | INSPEÇÃO DIMENSIONAL Armature (175) | Commutator Bar to Bar (Before Refinishing) Bar to bar: 0.0008”Max | VALOR NCC |  |  |
| Table 8002 §8.O | INSPEÇÃO DIMENSIONAL Armature (175) | Commutator Bar to Bar and Concentricity (After Refinishing) Bar to bar: 0.0001”Max TIR: 0.0005” Max | VALOR NCC |  |  |
| Table 8002 §8.O | INSPEÇÃO DIMENSIONAL Armature (175) | Mica Undercut (Before Re-cut) Depth: 0.045”- 0.070” Width: 0.035”-0.045” | VALOR NCC |  |  |
| Table 8002 §8.O | INSPEÇÃO DIMENSIONAL Armature (175) | Mica Undercut (After Re-cut) Depth: 0.050”- 0.070” Width: 0.035”-0.045” | VALOR NCC |  |  |
| Table 8002 §8.O | INSPEÇÃO DIMENSIONAL Armature (175) | Balance 5 grain inch Max. each end | VALOR NCC |  |  |
| Table 8002 §8.Q | INSPEÇÃO DIMENSIONAL Terminal Block (280) | Capacitor tolerance: Capacit. Values B and E 2.0µF +50%/-20% 1.0µA @50VDC | VALOR NCC |  |  |
| Table 8002 §8.Q | INSPEÇÃO DIMENSIONAL Terminal Block (280) | Capacitor tolerance: Capacit. Values B and ground 0.022µF +50%/-20% 1.0µA @50VDC | VALOR NCC |  |  |
| Table 8002 §8.Q | INSPEÇÃO DIMENSIONAL Terminal Block (280) | Capacitor tolerance: Capacit. Values C and ground 2.0µF +50%/-20% 1.0µA @50VDC | VALOR NCC |  |  |

| Item | Procedimentos | Critério | Resultado | Condição | Realizado por: |
| --- | --- | --- | --- | --- | --- |
| Item | Procedures | Criteria | Result | Condition | Performed by |
| Table 8002 §8.R.(4).(b) | INSPEÇÃO DIMENSIONAL Anti-drive end bell (270) | Bearing Liner Diameter: 1.8501” – 1.8504” | VALOR NCC |  |  |
| Table 8002 §8.T.1 | INSPEÇÃO DIMENSIONAL QAD kit end bell (365) | Dowel Pin projection (366): 0.115” minimum | VALOR NCC |  |  |

OBSERVAÇÕES / REMARKS

| Aprovador por: Approved by | I | Data: Date |  |
| --- | --- | --- | --- |

| RELAÇÃO DE RELATÓRIO / JOB REPORT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Responsável (Responsible) | Data (Date) | Folha (Sheet paper) | Nº Pag. (Page nº) |  | Responsável (Responsible) | Data (Date) | Folha (Sheet paper) | Nº Pag. (Page nº) |